██████╗░██╗░░░██╗████████╗███████╗

██╔══██╗╚██╗░██╔╝╚══██╔══╝██╔════╝

██████╦╝░╚████╔╝░░░░██║░░░█████╗░░

██╔══██╗░░╚██╔╝░░░░░██║░░░██╔══╝░░

██████╦╝░░░██║░░░░░░██║░░░███████╗

╚═════╝░░░░╚═╝░░░░░░╚═╝░░░╚══════╝

Electronics

Task 02 - Simon Says with Heartbeat

1.Speed ramp formula:

The speed ramp formula implemented here is a linear decrease of-

- The time the LED stays on

- The time between the current led and the successive one

This ensured that the game progresses in a reliable way while keeping tension high as-

- The sequence grows longer making the patter more difficult to remember

- The time between LED’s being reduced as well as the time LED stays on makes the game very snappy

In code these values are affected by the variables ‘GAME_SPEED’ and ‘diff’

2.Formula for staying independant:

The game cleverly stays independent of the heartbeat LED by avoiding the use of the delay() function and instead utilising the millis() function to capture time snapshots. The millis() function provides the time(in milliseconds) since the arduino started and hence can be used for calculations. The whole game logic runs on a series of if-loops that utilise flags to traverse throughout the game. This ensures the game loop can run simultaneously and flags can be used to switch what part of the game runs without interfering with the heartbeat loop.